NetWatch Release Notes

Version 5.0.0- Windows, NetWare, Unix, Linux, VMWare

These release notes are provided as a guide to help troubleshoot NetWatch

deployment issues. This document is not intended to be all-inclusive. The

contents of this document are copyright 2008 Eaton Corporation. For the latest updates

and product release information, visit www.powerware.com.

For additional installation and software assistance, review the NetWatch help

files.

--------------------------------------------------------------------------------

Table of Contents

1.0 Product Description

2.0 System Requirements

3.0 Compatible Devices

4.0 New Features

5.0 Known Issues

6.0 Workarounds

7.0 Bug Fixes

--------------------------------------------------------------------------------

1.0 Product Description

--------------------------------------------------------------------------------

The Netwatch Client software is provided to shut down a server or workstation under the control of a UPS being monitored by a ConnectUPS Web/SNMP card, BestLink Web/SNMP adapter or MGE Network Management Card (NMC).

Netwatch Clients monitor the activity of a single UPS through communication with the ConnectUPS, BestLink or NMC. Netwatch alerts the user(s) of a server or workstation whenever a power failure occurs. During an extended power failure, the ConnectUPS, BestLink or NMC informs all configured Netwatch Clients that it is time to shut down their respective operating systems, thus preparing them for the removal of AC power.

--------------------------------------------------------------------------------

2.0 Firewall User Notice

--------------------------------------------------------------------------------

NetWatch normally requires UDP ports 200, 2844 and 2845 for communicating with Connect UPS or BestLink card and ports 4679 and 4689 when communicating with a MGE NMC.

If your system uses a Firewall, then these ports must be unblocked.

See the 'Advanced Network Settings' help file topic for more information

about UDP port configuration.

--------------------------------------------------------------------------------

3.0 System Requirements

--------------------------------------------------------------------------------

3.1 Supported Platforms

Operating systems are running on 32-bit processors unless otherwise specified.

* Windows

> Windows XP (&)

- Home

- Professional

> Windows Server 2003 (&)

- Standard

- Web

- Enterprise

> Windows Server 2008 (&)

> Windows Vista (&)

- Home Premium

- Business

- Ultimate

* HP-UX

> 10.20, 11.0, 11i (11.11) for PA-RISC (&)

> 11i v1.6 (11.22) for Itanium (&&)

> 11i v2 (11.23) for Itanium (&&)

* IBM AIX

> 5.1, 5.2 for PowerPC

> 5.3, 6.1 for Power5 (&&)

* Linux (Systems with Libc 2.3.x, 2.4.x, and 2.5.x are supported,

systems with Libc 2.2.x are not supported)

> Red Hat

- Red Hat Enterprise Linux v. 3 (&)

- Red Hat Enterprise Linux ES v. 4 (&)

- Red Hat Enterprise Linux ES v 4 for Itanium (&&)

- Red Hat Enterprise Linux AS v 4 for Itanium (&&)

> SuSe

- 9.x, 10.x (&)

- SuSe Enterprise Linux Server 9 (&)

> Fedora Core 5, 6

> Ubuntu Linux 6.10

* VMWare ESX Server

> VMWare ESX Server 3.5 (Linux Kernel 2.4)

* Novell NetWare

> 5.0, 5.1, 6.0, 6.5 (Must upgrade to the latest service packs)

* SCO OpenServer

> 5.0.7

* SGI Irix (MIPS)

> 6.5.2.x  (&&)

* Sun Solaris

> 8, 9, 10 for SPARC

> 8, 9, 10 for x86

& - Includes support for 32 and 64 bit processors

&& - Running on a 64 bit processor

Note: You should install the latest service packs and patches for all of the

operating systems listed above.

3.2 Windows Minimum Requirements

* Installed TCP/IP networking

* 200 MHz processor

* 20MB of RAM

* 30MB of available hard-disk space

* CD-ROM drive, if installing from CD

3.3 IBM AIX Minimum Requirements

* Installed TCP/IP networking

* 66 MHz processor

* 16 MB of RAM

* 16 MB of available hard-disk space

3.6 HP-UX Minimum Requirements

* Installed TCP/IP networking

* 66 MHz processor

* 16 MB of RAM

* 16 MB of available hard-disk space

3.7 Linux Minimum Requirements

* Installed TCP/IP networking

* 66 MHz processor

* 32 MB of RAM

* 16 MB of available hard-disk space

3.8 Mac OS Minimum Requirements

* Installed TCP/IP networking

* 500 MHz processor

* 32 MB of RAM

* 16 MB of available hard-disk space

3.9 NetWare Minimum Requirements

* Installed TCP/IP networking

* 200 MHz processor

* 16 MB of RAM

* 16 MB of available hard-disk space

3.10 SCO OpenServer Minimum Requirements

* Installed TCP/IP networking

* 66 MHz processor

* 32 MB of RAM

* 16 MB of available hard-disk space

3.11 SGI IRIX Minimum Requirements

* Installed TCP/IP networking

* 66 MHz processor

* 32 MB of RAM

* 16 MB of available hard-disk space

3.12 Solaris Minimum Requirements

* Installed TCP/IP networking

* 70 MHz processor

* 16 MB of RAM

* 16 MB of available hard-disk space

--------------------------------------------------------------------------------

4.0 Compatible Devices

--------------------------------------------------------------------------------

The following have been tested to work with NetWatch

4.1 Communication Cards

* ConnectUPS-X Web/SNMP 10/100 Card (X-Slot)

* ConnectUPS-BD Web/SNMP 10/100 Card (X-Slot)

* ConnectUPS-E Web/SNMP network adapter

* MGE Network Management Card 66102

(Note: MGE NMC Firmware must be at version DA or above). (http://download.mgeops.com/emb/htm/66102e.htm)

Also refer to section 9.0 for Limitations related to NMC 66102 support

--------------------------------------------------------------------------------

5.0 New Features

--------------------------------------------------------------------------------

5.0.0

- Added support for MGE Network Management Card (66102)

- Added support for AIX 6

- Added support for Fedora Core 6

- Added support for Windows Server 2008

- Added support for VMWare Server ESX 3.5

- Added support for Solaris 10 (SPARC and Intel)

- Dropped support for Fedora Core 2

- Dropped support for IBM AIX 4.3.x

- Dropped support for Red Hat Linux v 9.x

- Dropped support for Red Hat Enterprise Linux AS v2.1

- Dropped support for Sun Solaris v7 (SPARC and Intel)

4.2.0

- Added support for Windows Vista.

- Added support for Red Hat AS, ES for Itanium2.

- Added support for Fedora Core 5.

- Added support for Ubuntu Linux 6.10.

4.1.2

- Batch script execution has been decoupled from the NetWatch system shutdown

process on UNIX and Windows.

- Added batch script support for UNIX.

- Added support for HP-UX Itanium systems.

- Added support for Suse 9.x & 10.x.

- Added support for embedded PPC Linux.

- Added support for Red Hat Enterprise Linux ES v. 4.

- Dropped support for IBM AIX 4.1.X.

4.1.1

- Added Windows 2003 Installation support.

Previous version of the NetWatch installation did not explicitly support

Windows 2003.

4.1.0

- New features for NetWatch v4.1.0.7 include support

for systems with multiple network interface cards (NICs).

- Removed the option to specify an alternate installation.

- Default installation path was changed from "/etc/powerware/netwatch"

to "/usr/Powerware/NetWatch".

- The install script will now warn the user if a previous NetWatch installation

was detected. The user can select to uninstall the previous NetWatch

installation and continue with the new, or exit the install script. All old

files, links, and directories will be removed regardless if the old

installation was located in

"/etc/powerware/netwatch" or "/usr/Powerware/NetWatch".

--------------------------------------------------------------------------------

6.0 Known Issues

--------------------------------------------------------------------------------

ITEM	      : NetWatch does not run batch files before shutdown

ITEM #     : 2406

DESCRIPTION: On the Linux client, there is an option to run a batch file, nw_batch.sh before OS is shut-down. This batch file does not run on shutdown even if it is enabled.

WORKAROUND: None

ITEM       : Open documents are not saved/backup during NetWatch initiated shutdown

ITEM #     : 2368

DESCRIPTION:  Netwatch should save all the open documents before shutting down the Operating system. But it fails to do that and does not save open documents and instead shows dialog that windows will restart after some time.

WORKAROUND : None

ITEM       : Unix / Linux Command Options do not work

ITEM #     : 2159

DESCRIPTION: Under Unix and Linux, you can configure NetWatch from the command prompt. When the NetWatch service is running, the options set by those commands do not get set.

WORKAROUND : You need to stop the NetWatch service by  (./netwatch.init stop), use commands to set options and then restart the NetWatch (./netwatch.init start).

ITEM       : After installing NetWatch, some Unix/Linux machines to not release the CD.

ITEM #     : 1672

DESCRIPTION: Following installation of NetWatch from a CD, on some Unix/Linux machines do not return to a command prompt.  , after installation is completed the control does not come to prompt and seems like hang.

WORKAROUND : Come out of command prompt using Cntrl+C and, copy Unix/Linux install files locally to hard disk and run it from there.

--------------------------------------------------------------------------------

7.0 Tips

- Unix Uninstall Fails to Remove the Powerware & NetWatch folders

When running uninstall.sh from the directory where NetWatch was installed (by default, it is /usr/Powerware/NetWatch),  the Powerware and NetWatch folders are not removed.

Workaround: Run the uninstall.sh from outside the directory where NetWatch was installed.

- Unable to find MGE UPSs in the network

Check the firmware version for the Network Management Card, it should be vDA and above.

- Unable to find one or more UPSs in the network

Check if the firewall is On. If firewall is ON, please enable the ports used by NetWatch. 1. For Powerware ConnectUPS card, default is 2844 and 2845 or configured ports should be allowed.

2. For MGE Network Management Cards, the ports 4679 and 4680 should be allowed.

-MGE UPSs in Different Subnet

Router/ switch ports 4679 and 4680 need to be configured if NetWatch and MGS UPS’s are in different subnets

- Netwatch Configurator on Vista

Incase UAC(User Access Control located in control panel\User Accounts\Turn User

account control ON or OFF) is ON, then run netwatch configurator in Administrator

mode (right click on NetWatch configurator and select "run as administrator").

Incase UAC is OFF then run NetWatch configurator in normal mode.

- Event Messaging

On certain Unix/Linux based operating systems, messages for a communications failure

do not appear in the console although they appear in the logs. Critical

messages such as shutdown notifications are displayed on all open TTY consoles.

Informational messages are sent to the system console. If informational

messages are not being displayed, open the system console or designate any

console as the system console.

- Unexpected long delay for connection message for OFF load segment or Communication Failure

NetWatch polls network adapters once every 5 minutes for status. In order to prevent erroneous communication failure warnings, NetWatch contacts a network adapter 3 times before signaling a communication failure. This can result in a communications failure not being reported for 10 to 15 minutes.

--------------------------------------------------------------------------------

8.0 Bug Fixes

--------------------------------------------------------------------------------

5.0.0

- Enable Operating System Shutdown after ____ minutes power failure" cannot change

interval on first pass

Change interval box was not behaving properly. It will now become enable when

selecting check box.

- Netwatch Registry had invalid settings

NetWatch Registry updated to show correct settings

- Netwatch Window shows incorrect Segment of the Current shutdown Server.

When opened Netwatch Configuration Utility, it is not showing correct Load segment selected previously.

This issue is now fixed and it will show exact load segment selected previously.

4.1.2

- Linux Enterprise Linux ES & AS

Fixed 32/64 bit system structure alignment issues.

- UNIX Installation

The UNIX install script will immediately exit if the local UNIX OS variant

is not supported.

- HP-UX Installation

Updated UNIX install script to detect HP-UX systems where the 'uname -a' command

doesn't always produce a 'HP-UX' text occurance in the result string.

- Linux Installation

Updated UNIX install script to detect the Linux Libc version and install the

appropriate binary file.

- Fixed configuration file read problem for the "Use NAT" parameter.

- Added support for Improved communcations lost check to filter out

spurious events.

- Improved communcations lost check to filter out spurious events.

- Windows Shutdown Issues

Improved Windows shutdown process to forcefully terminate stubborn programs

and allow Windows to gracefully shutdown.

4.1.1

- Fixed socket error issue on UNIX. Systems with with multiple NICs could cause

NetWatch to fail prematurely when registering to the ConnectUPS.

- Fixed Autorun problem on Windows 98/ME.

- UNIX HTML help page had several incorrect links.

- Windows Installation Crash

Fixed a crash problem that would occasionally occur when the install

was started.

- Windows Installation Error

Fixed a 'Component Transfer Error' problem with Windows 2003.

- Linux Installation Problem

Fixed 'Bad syntax' message when installing on SUSE Linux.

- UNIX Installation Issues

Cleaned up minor problems with the install and uninstall scripting.

- TCP/IP vs UDP in documentation

Changed all references of 'TCP/IP' to 'UDP' in documentation and programs.

This was done so that users understand NetWatch uses UDP for network

communications, and can set up firewalls accordingly.

- Installation - UNIX

Revised install script to handle tar files in upper, lower, or mixed cased file

name format.

- SVR4 Installation Problem

Fixed 'id -u' script problem when installing on SVR4 systems.

- Control Panel - Win32

The IP common dialog control has been replaced by an edit box to handle dotted

IP addresses or host names when using the manual IP dialog box. The IP common

dialog control also had incorrectly converted '0's to '255'.

4.1.0.7

- Installation - UNIX

Resolved problems that would prevent users from installing or reinstalling the

software if a previous installation was present on the system. Also fixed some

problems in the reinstall process.

- Uninstall - UNIX

Fixed a problem that would cause the NetWatch uninstall script to exit

prematurely.

--------------------------------------------------------------------------------

9.0 Limitations

- “OS Shutdown Controlled by Shutdown Server” is not available on MGE Network Management Card 66102  So user should select “Timed Shutdown” while configuring NetWatch v5.0 for MGE NMC card