TROUBLESHOOTING

===============

I hope most people will have no trouble with the SCRAM 500, but

obviously some boards will struggle to life or one day myseriously die.

The tips described here should be used as a guide, however, tricky

problems will come up that need a reasonable amount of diagnostic skill.

Hopefully, someone in your city or User Group or on the Nets will be

able to help. Armed with the Schematic Diagrams, just about any

AmigaTechnoPerson should be able to locate the fault. At worst they

might need an oscilloscope and some beer.

Stay cool, think positive and DON'T PANIC!

Catastrophe - smoke!

--------------------

Let's deal with the really scary scenario first - you blow up the Amiga.

This isn't as bad as it might seem. I haven't seen this happen often and

when it does happen it usually is only the 68000 chip that is dead. That

is easily fixed, and probably gives you an excuse to upgrade Agnus or

KickStart any way. If you do replace the 68000, you might put in a

HC68000 device - HC uses less power and is a lot cleaner. Before you

blow it up again, try to work out why it happened. Here are some ideas.

o You plugged/unplugged the SCRAM with power on.

o You zapped the system with static - watch out for carpets.

o Your SCRAM board has a serious short (power to signal, for example)

Next on the gloom scale, the SCRAM starts to smoke. This can be serious,

but usually it can be fixed. You probably haven't checked the board

thoroughly for shorts and bridges. Now the problem will be highlighted

for you in carbon! If the SCRAM has been checked out with a multi-meter,

then you should be able to avoid this unhappy circumstance. The other

likely cause of smoke is an IC back to front. This will usually require

replacing the IC with a new one.

The SCRAM stops the Amiga from working

--------------------------------------

You plug the SCRAM on and the Amiga is dead - just a black screen. This

means the SCRAM is seriously interfering with some vital Amiga function.

Go through the following check list.

o remove all socketed chips from SCRAM - any change?

o check power rails for normal voltages

o check expansion bus signals for funny levels (logic probe / Scope)

o recheck SCRAM for solder bridges and gross faults

o last resort - start cutting TTL out (avoid if possible)

Fixing this type of fault requires some diagnostic skills - the cause is

likely to be different depending on whether the board has just been

built or has failed in service. If you pull out all socketed devices and

the problem remains, it is almost certainly a solder bridge.

The Amiga only gets to a coloured screen

----------------------------------------

This is a slight advance on the previous fault. This is more likely to

be an address or data bus fault (typically a short). Remove all socketed

chips and look for shorts.

The SCRAM Config LED flashes

----------------------------

Things are getting better. The SCRAM Config LED is turned on when

KickStart goes out to AutoConfig expansion boards. This means the 68000

is getting a fair way along, and the SCRAM is mainly working. Again,

remove socketed chips and see if the Amiga functions normally. If it

does, then the fault is probably not an address/data short, but some

more benign fault. Try the following techniques:-

Disable the RAM part of the board.

o Pull out all DRAMs (they shouldn't be in yet anyway)

o Set jumpers for 0Megs RAM

o Remove Humphrey and Griswold chips

The RAM section of the board will now be inactive. Only a gross fault

(ie solder short) can be causing trouble.

Disable the SCSI controller

o Pull out Boot ROM, Cyril and SCSI chip.

o Disable AutoBoot switch

The SCSI section is now inoperative and should not be causing

problems.

With Bertie in place, you should be able to get the Config LED to come

on and stay on when you start the Amiga. This should now be your goal.

If you study the schematic (sheet 1/3) you will see that the

AutoConfig logic is fairly simple - most stuff is happening inside

Bertie. Once you get AutoConfig happening, you can proceed with the

other sections by populating them and testing them.

SCRAM configs, but RAM appears to be faulty

-------------------------------------------

If RAM only partially works, it is likely to be one of the following:-

o solder faults in the ZIP array - dry joints and bridges!

o bent RAM pin in socket

o error with the LS257 multiplexors

o faulty RAM chip

To diagnose RAM faults try RAM TEST programs, Monitor programs and a bit

of detective work. For example, if you can get a Monitor working, you

can tell a lot by writing patterns of 00 and FF to all the RAM

o one of the 4 nibbles not working -

? faulty DRAM chip

? missing +5, GND, RAS, CAS

o repeating blocks not working or duplicated

? RAM mux address line missing or shorted

? solder fault on LS257 or ZIP array

o RAM corrupts with time (refresh problem)

? missing E clk

? check 393 device

? check REFRAS\ signal from Griswold to Humphrey

If you can't get the RAM to do anything, then look at the signals

supposedly emenating from Griswold and Humphrey. You should see

reasonable RAS, CAS and REFRESH signals. If you don't, then suspect one

of these chips.  The easiest way to debug the Custom Chips is swapping

with known good chips. Having access to a working SCRAM is also useful

for debugging.

SCSI Problems

-------------

The SCSI circuit is simple and socketed. Just about any problem will

look the same - the disk won't talk!

Make sure your SCSI drive is working and the cables are good. Don't use

25 way ribbon to connect to SCSI - scrambled wires or 50 Way Ribbon is

important to avoid crosstalk problems. Some of the new SCSI devices have

very high transfer rates and can be cranky with long or noisy cables.

Look for all the normal solder problems - dry joints, bridges etc.

Sometimes PLCC chips can have damaged or bent pins so check this.

Use a Monitor program to try to read the EPROM and SCSI registers.

Try to activate the Disk LED - prove Cyril is working.

Replace SCSI chip (static sensitive device)

It should be fairly easy to write a bit of code that goes out and does

some diagnostics on the SCSI chip to find problems. All the

documentation on the DP8490 chip is available in the National

Semiconductor Mass Storage Handbook. Some of this is SCRAM 500 User's

Manual.

SUPPORT

-------

I hope that as people get SCRAMs going they become sources of support

for others. There should be a reasonable amount of healthy discussion on

the nets which may provide encouragement for the owners of non-working

SCRAMs.

If you have a serious problem you can send me an EMAIL, and I will try

to solve it (the problem), but try other more direct avenues first.

EMAIL normj@runx.oz.au

In Australia, we have a resident SCRAM expert who can fix all things

Amiga.

Chris Ralph, Sydney - Phone (02) 557 1050