# Victrack Access

# Trans-Mit Pty Ltd

Quotation to migrate the Telmax21

to VMWare

# Contact:  Rod Sheppard

# TELEPHONE:  03 9574 8956

# EMAIL:  rod@transmit.com.au

# WEBSITE:  www.transmit.com.au

Project Background

Victrack is seeking a quotation to migrate the Telmax21 to VMWare.

Scope of Work

- Load, install and configure Telmax21 with existing the existing OS software (SCO 6.0.0) and Database licenses.

- Transfer the database, and applications software and data from the existing server

- Install and configure software for backup of the Telmax21.

- Impliment security changes, including. OS patches, modifications to  Apache and other configuration changes to satisfy security scans.

- Supply dual Serial over Ethernet devices, to allow dialup polling from the VM. The modems are to be supplied by Victrack. (Existing ones could be used.)

*** If the serial over Ethernet device with built in modems are chosen. Then the modems will not be required.

- Configure the Telmax21 to poll using both devices. Preferable to cycle between them.

- Test and commission

Telmax21 Server Operating System Software Upgrade

Installation

The SCO UNIX operating system will be installed and configured. An IP address (different to the current server) will be required. The Database software will be installed and configured. The base Telmax21 applications will also be installed.

Transmit will create the virtual machine at the Transmit Office and send to Victrack either by upload or DVD media.

During commissioning, the configured applications, Informix database and Bilmax21 data will be transferred from the existing machine or backup server.

Security changes will be implemented at this time.

After installation it is assumed that Transmit will have access to the virtual machine via ssh for remote diagnostics support and health checks.

If Trans-Mit is required to attend site then a minimum $375 would apply and any time in excess of 3 hours will be charged at $125 per hour or part thereof.

##### Pricing and Delivery

- Installation and configuration of SCO Openserver, 6.0.0 on the virtual machine. Security patches/configuration. Installation and configuration of Informix database. Software. Installation and configuration of Telmax21 applications and backup software, plus data transfer, test and commission. Setup of dual polling serial over ethernet

.			 	$3,650

- Serial over Ethernet Devices

I have investigated the various possibilities for the serial over ethernet devices, you will need to choose from the following These are the dealer prices, which I will pass on to you without any markup.

From Perle…

Single Port IOLAN DS1:04030124

IOLAN DS1  1 x DB9M connector,  software selectable, EIA-232/422/485 interface,10/100 Ethernet, IPv6,COM port redirector, 15kv

2 x $232

3 Port (with In-built Modem
04030826 IOLAN SDS3 M  3 x RJ45 connector with one integrated V.92/V.90 data modem, software selectable EIA232/422/485 interface,10/100 Ethernet, advanced data
encryption, user authentication and event management security

2 x $794

This is the smallest Rack Mount unit that Perle provide:
04030316 IOLAN SDS8 8 x RJ45 connector , 1U rack mount, software selectable EIA232/422/485 interface, 10/100/1000 Ethernet, advanced data encryption, user
authentication and event management security features included, IPv6, COM port redirector, 15kv

2 x $1,410

- Serial over Ethernet Devices (cont.)

From MPA…

I haven’t managed to get specs from them, but have verbal pricing information. They cannot supply the Equinox Units as previously suggested. They are now supplying SnapGear.

SnapGear 1 Port Serial over Ethernet (I believe it is wall mount)

2 x $396

SnapGear16 Port Serial over Ethernet (rack mount)

2 x $896

- Optional.

Transmit to buy back the no longer needed OS upgrade licenses for SCO Openserver 5.0.6 to 6.0.0. 																	 ($500)

Notes:

- All prices are exclusive of GST

- Delivery 2 to 4 weeks after receipt of order.

- Quotation is valid for 30 days.

4.  	If you choose to supply your own units, then they must be SCO Unix Openserver  6.0.0 compatible